1. Power Spectral Density (PSD) - Welch Method

Purpose

The Welch PSD estimate shows how signal power is distributed across frequency before and after processing. It is useful for verifying:

- Noise reduction

- Preservation of desired frequency components

- Suppression of interference bands

Method

Use Welch’s averaged periodogram:

Pxx(f)=1LUl=0L-1∣n=0N-1xl(n)w(n)e-j2πfn∣2

where:

- w(n)= window function

- L= number of segments

- U= normalization factor

What to Show

Plot PSD curves for:

- Noisy signal

- Processed signal

- (Optional) clean reference signal

Interpretation

Effective processing should show:

- Reduced broadband noise floor

- Removal of unwanted peaks/interference

- Retention of dominant signal frequencies

2. Spectrogram (STFT)

Purpose

A spectrogram provides a time–frequency representation and reveals:

- Transient noise

- Nonstationary interference

- Temporal preservation of signal structure

STFT Expression

X(m,ω)=n=-∞∞x[n]w[n-m]e-jωn

What to Compare

Display side-by-side spectrograms:

- Original noisy signal

- Enhanced/filtered signal

Desired Observations

A successful method should:

- Reduce diffuse noise energy

- Preserve harmonics/formants/transients

- Maintain temporal continuity

3. Quantitative Indicators

Signal-to-Noise Ratio (SNR)

Formula

SNR=10log⁡10PsignalPnoise dB

SNR Improvement

ΔSNR=SNRout-SNRin

Interpretation

Higher positive values indicate better noise suppression.

4. Distortion Analysis

Even when noise is reduced, processing may introduce artifacts.

Common Distortions

Amplitude Distortion

- Attenuation of useful signal components

- Over-smoothing

Phase Distortion

- Temporal misalignment

- Smearing of transients

Musical Noise / Artifacts

Common in spectral subtraction methods.

Loss of High-Frequency Components

Can reduce intelligibility or sharpness.

5. Possible Limitations

Discuss practical limitations honestly.

Typical Limitations

Nonstationary Noise

Performance may degrade when noise characteristics vary rapidly.

Parameter Sensitivity

Window length, overlap, and thresholds affect results.

Computational Complexity

STFT- or adaptive-based methods may be unsuitable for real-time systems.

Trade-Off Between Noise Reduction and Distortion

Aggressive filtering often removes useful signal content.

Dependence on Accurate Noise Estimation

Incorrect noise models reduce effectiveness.